Data Sheet

KEN Washer Disinfector IQ3

Standard Range Features - OLED Display with soft touch program keys on control panel - Open Programs Structure - Very low water consumption - Low Noise Levels - Low Energy Costs - Maintenance free sliding doors - Service access from the front - EN ISO 15883-1-2-5-6 compliant - Glass surfaces easy to clean.

| IQ3 |  |
| --- | --- |
| Outer dimensions Outer dimensions with side cabinet | H: 840 x W: 600 x D: 630 mm H: 840 x W: 900 x D: 630 mm |
| Wash chamber dimensions | H: 490 x W: 540 x D: 556 mm |
| Operative volume, wash chamber | 150 Litres |
| Total chamber volume | 170 Litres |
| Capacity of DIN baskets (480x250x50mm) | 4x2 DIN Baskets |
| Water consumption per phase (4 level rack) | 8-14 litres |
| Programs | 10 Standard + 30 optional |
| Process times | Standard 40-60 min. |
| Water connection | 1 x ¾” (2 x ¾” are optional) |
| Cold water Pressure | 2-8 Bar, ≥15L/min, 5-30˚, 0-30˚dH |
| Hot water | 2-8 Bar, ≥15L/min, 30-70˚, 0-3˚dH |
| Demineralised/RO water | 2-8 Bar, 15L/min 5-70˚ cond. 0-200μS |
| Condenser | Standard |

* The above is the standard specification. Any modification/deviation for the above will be clearly identified on the quotation front sheet.

Additional Options and Accessories:			XML server Connection

Printer

2 x Additional Water Connections

Drying Module